Cisco Enable WPA for Tablets and Verifone PEDS

CISCO AIR SAP1602I - SETUP FROM SCRATCH

Turn on the AP

- Boot up the AP – Find the IP address of the AP it will default to DHCP so you need to find out what it is.

Connect to AP

- Connect using HTTP://<IP OF ACCESS POINT>

user= Cisco		- note CAPITAL C

pass = Cisco

Configure AP Using Web Interface

Step 1 – SSID Config

1. Goto HOME

2. Easy Setup

3. Network Configuration

4. SSID for  Radio 2.4GHz and Radio 5GHz enter IBSAP1

5. Tick Broadcast SSID in Beacon on both

6. PRSSS APPLY under Radio 2.4GHz

Step 2 – Radio Enable

1. Goto Security TAB

2. Select SSID Manager

3. SELECT IBSAP1

4. Interface - Tick Radio0-802.11N2.4GHz + Radio1-802.11N5GHz

5. PRESS APPLY

Step 3 – Encryption Mode

1. Goto Security TAB

2. Select Encryption Manager

3. Cipher - Encryption Mode Set To AES CCMP + TKIP

4. Press - APPLY ALL

Step 4 – Auth And PSK

1. Goto Security TAB

2. Select SSID Manager

3. Select IBSAP1 SSID from the Current SSID List

4. Select "Open Authentication" Select "<NO ADDITION>"

5. Set Key Management to Mandatory

6. Check the Enable WPA checkbox

7. Select WPA from the drop-down list

8. Enter the WPA Pre-shared Key. - 36277d354e

9. PRESS APPLY

Step 5 – Enable Radio

Select HOME

select Radio0-802.11N2.4GHz

select settings

select ENABLE

PRESS APPLY

Select HOME

select Radio1-802.11N5GHz

select settings

select ENABLE

PRESS APPLY

Refresh a few time both Radios should be UP and GREEN - may take a while

Step 7 – AP Security

Select SECURITY

Select ADMIN ACCESS

Highlight NEW Above "Cisco"

Enter admin in Username

Enter 3Hevron5 in Password

Capability Settings: = Read-Write

PRESS APPLY

Select CISCO

Press "DELETE"

Step 8 – Set User List

Select Local User List Only (Individual Passwords)

PRESS APPLY

Final Settings Info

Login to AP as admin - 3Hevron5

The SSID for Connection will be IBSAP1

The Password for Connection will be 36277d354e

Encryption is WPA